# How to take pictures for Gaussian

1. Core principles for Gaussian splats

Across different guides and tools, the same 3 rules keep coming back:

- Consistency

  - Fixed focal length.

  - Lock exposure, ISO and white balance.

  - Avoid auto focus hunting; ideally use manual focus once and keep it.

- Clarity

  - No motion blur: use fast shutter (e.g. 1/250+ on handheld stills, higher for video).

  - Depth of field (f/5.6–f/11 on crop/FF, or “everything in focus” on phone).

  - No rolling-shutter wobble from whipping the camera around.

- Coverage & parallax

  - Move the camera through space, don’t just rotate in place.

  - Loads of overlapping views from different positions and heights.

  - Include some background/room context, not just the object.

2. Camera settings (DSLR / mirrorless / phone)

Based on Reflct’s 3DGS capture tips, SmartDataScan’s gsplat tutorial, and Volinga’s field guide:

Mode

- Shoot stills if you want best quality (less motion blur, more control).

- Use video + frame extraction if you want speed or are walking a long street. Record at 60 fps if possible and later extract 1 sharp frame every ~0.5 s

- 1/6 -

Manual settings (ideal)

- Focal length: moderately wide (full-frame equiv. ~24–35 mm). Avoid extreme wide.

- Aperture: keep the scene sharp:

  - APS-C / FF: around f/5.6–f/11.

- Shutter: as fast as light allows:

  - Handheld stills: ≥1/250 s (slower only if you use a tripod).

  - Video: shutter ~2× frame rate (1/120 s for 60 fps) to avoid smear.

- ISO: pick a fixed value (ex: 100–800); NO auto ISO.

- White balance: lock WB (ex: daylight / cloudy / 4500 K), never auto.

- Focus:

  - Prefer manual focus: pre-focus on your subject and leave it.

  - If you must use AF, turn off continuous refocus and refocus only occasionally.

Phone-specific hacks

If using a smartphone:

- Turn off “night mode,” HDR bursts or any fancy multi-frame modes if they cause ghosts.

- Lock exposure and WB if the camera app allows.

- Move very smooth and slow because phones add rolling-shutter wobble.

3. How much overlap & how many images?

Different sources give slightly different numbers:

- SmartDataScan: 20–30% overlap minimum between adjacent shots.

- Reflct (for stills): recommends 70–80% overlap for robust parallax.

- 2/6 -

You can think of it like this:

- Video: you automatically get tons of overlap, so just move steadily.

- Stills: aim for something in the 50–80% overlap range; it’s practically “small steps, lots of redundancy”.

Typical image counts from real 3DGS capture guides:

- Small object: approx. 100–200 images.

- Small room:  approx. 150–300 images, depending on clutter.

- Larger street corner / façade: often 150–400 images.

- More is not always better! Thousands of near-identical frames can hurt (overfitting, longer training). Reflct warns about >80% overlap and redundant views.

4. Movement patterns (this is where it differs from “single orbit” photogrammetry)

From Volinga and Reflct: you must translate, and you must create parallax.

- Don’t stand in one spot and just pan.

- Move the camera through space:

  - Walk paths.

  - Change height.

  - Mix near and far distances.

- 3/6 -

Think in layers instead of a single orbit:

- One circle rotation closer, one mid, one further away.

- Some shots from above, some from lower angles.

- Some tangential/oblique views of edges and corners.

5. Concrete recipes for our use-cases

A. Street corner (about 100–300 images)

Use a walkthrough pattern rather than just circling one object.

- Plan the area

  - Decide what must be perfect (ex: building facade) vs. just context.

  - Avoid rush hour / heavy pedestrian traffic, if possible, the scope is obtaining a static scene.

- Main pass: building side A

  - Start 5–10 m away from the building, camera at eye-level.

  - Walk parallel to the facade, taking a shot every around 0.5–1 m (or record video).

  - Slightly angle the camera toward the building so it’s always in frame.

- Second pass: opposite side / diagonals

  - Cross the street (if safe) and repeat, so you have a second viewpoint band.

  - Add a third pass where you walk diagonally across the corner, to give strong parallax on the corner edges.

- Height variation

  - Do another lap while holding the camera higher above your head, and one with the camera slightly lower (chest/stomach level).

  - Tilt up/down a bit to cover windows, roofline, and ground.

- Detail passes

  - Move closer to important features (doorway, statue) and do a tight mini orbit around them while keeping some background visible for COLMAP.

- 4/6 -

IMPORTANT

- Avoid cars/people moving through the frame; if someone walks through, just skip that frame when you build your dataset.

- Try not to have big glass surfaces dominating every frame

B. Small room (about 160+ images)

Combining a three-row eye-level strategy with Reflct’s room recipe:

- Perimeter passes

  - 2–3 loops around the room close to the walls:

    - Loop 1: camera at chest height, lens facing the wall, distance something around 1–2 m.

    - Loop 2: camera higher (eye / above head), angled slightly down.

    - Optional Loop 3: camera lower, angled slightly up.

  - Take a shot every 0.5–1 m along the wall.

- Center passes

  - Stand near the middle, then walk small arcs while looking toward each wall.

  - Mix distances: some shots closer to furniture, some more centered.

  - This gives cross-room parallax, not just “wall-hugging” shots.

- Floor and ceiling

  - For floor: hold the camera high, point it down 30–60°, and walk a grid.

  - For ceiling: hold it lower, point up, and repeat a grid-like walk.

  - There is no need for crazy density, but there must be enough shots, so the model sees from above/below, not only horizontally.

- Deal with white / plain walls

  - Textureless walls are hard for COLMAP and Gaussian splats (they mention this explicitly).

  - Add subtle texture: posters, painter’s tape markers, or let some furniture overlap into the wall shots.

160 images are totally reasonable for a modest room with this pattern.

- 5/6 -

6. Lighting:

All the serious guides say the same thing: even, soft and boring light is the best.

- Cloudy daylight, shade, or diffused artificial light.

- Avoid:

  - Strong directional sunlight.

  - Moving shadows (people, trees, traffic).

  - Flickering neon / LED patterns.

- Indoors: Avoid tiny bright windows + dark corners (huge dynamic range). Either add fill light or choose a time of day with more balanced light.

7. Differences vs the classic photogrammetry “orbit”

- Don’t stop at one orbit

  - Do multiple rings at different heights and distances instead of one perfect circle.

- Think “environment,” not just “object”

  - 3DGS shines when you include surroundings: floors, ceilings, background buildings, etc.

- Be stricter with consistency

  - For Gaussian splats, inconsistency with auto exposure/WB often shows up as weird color/noise artifacts in the final scene.

8. References

https://web.volinga.ai/capturing-gaussian-splats-blog/

https://smartdatascan.com/tutorials/gaussian-splatting-windows/capturing/

https://docs.reflct.app/capturing_gaussian_splats

https://medium.com/%40heyulei/capture-images-for-gaussian-splatting-81d081bbc826

https://www.creativeailab.be/beginners-guide-to-gaussian-splatting/

- 6/6 -